15. Write 8085 Program 8 bit multiplication.

LDA 2050H    	: Load the first 8-bit number (Multiplicand) into Accumulator

MOV B, A     	: Move the multiplicand from Accumulator to Register B

LDA 2051H    	: Load the second 8-bit number (Multiplier) into Accumulator

MOV C, A    	: Move the multiplier to Register C (serves as the loop counter)

MVI A, 00H   	: Clear Accumulator (A = 00H) to store the lower byte of sum

MVI D, 00H   	: Clear Register D (D = 00H) to store the higher byte (Carry)

LOOP:

ADD B        	: Add the multiplicand (B) to the Accumulator (A = A + B)

JNC SKIP     	: If there is no carry (CY = 0), skip the next instruction

INR D        	: If a carry is generated, increment Register D by 1

SKIP:

DCR C        	: Decrement the loop counter in Register C (C = C - 1)

JNZ LOOP     	: If Register C is not zero, loop back to perform addition again

STA 3050H    	: Store the lower 8 bits of the product (from Accumulator) at 3050H

MOV A, D     	: Move the higher 8 bits of the product (from Register D) to Accumulator

STA 3051H    	: Store the higher 8 bits at 3051H

HLT          	: Terminate/Halt the program execution