12. Design and implement the 8:1 MUX using gates /ICs

8:1 MULTIPLEXER

AIM: To verify the truth table of Multiplexer.

Theory:

The TTL/MSI SN54/74LS151 is a high speed 8-input Digital Multiplexer. It provides, in one

package, the ability to select one bit of data from up to eight sources. The LS151 can be used as universal function generator to generate any logic function of 4 variables. Both assertion and negation outputs are provided.

 Schottky Process for high speed

 Multifunction capability

 On-chip select logic decoding.

 Fully buffered complementary outputs

 Input clam diodes limit high speed termination effects.

Functional description:

The LS151 is a logical implementation of a single pole, 8-position switch with the switch

position controlled by the state of three select inputs, s0, s1, s2. Both assertion and negation

outputs are provided. The enable input (E) is active LOW. When it is not activated, the negation

output is HIGH and assertion output is LOW regardless of all other inputs.

PROCEDURE:

1. Connect Inputs S2, S1, S0, I0, I1, I2, I3, I4, I5, I6, I7 and Enable (E) to Logic Input Sockets.

2. Connect Output terminal Z &̅ Z to Logic Output Sockets.

3. Verify output with given Truth Table.

CONCLUSION: Hence verified the truth table of Multiplexer.

13. Write 8085 Program 8-bit addition

MVI C, 00H 	: Initialize Register C to 00H to store Carry

LDA 2500H 	: Load the 1st number from memory location 2500H into Accumulator

MOV B, A  	: Move the 1st number from Accumulator to Register B

LDA 2501H 	: Load the 2nd number from memory location 2501H into Accumulator

ADD B     		: Add contents of Register B to the Accumulator (A = A + B)

JNC SAVE    	: If Carry flag = 0 (No Carry), jump to SAVE

INR C      	 	: If Carry flag = 1, increment Register C by 1

SAVE:

STA 2502H  	: Store the lower 8-bit sum from Accumulator to 2502H

MOV A, C    	: Move the Carry value from Register C to Accumulator

STA 2503H  	: Store the Carry value from Accumulator to 2503H

HLT           	: Halt/Terminate the program execution

14. Write 8085 Program 8-bit Subtraction

ORG 2000H        : Starting address of the program

MVI C, 00H        : Initialize Register C to 00H to track borrow status

LHLD 2500H      : Load HL register pair: L gets data from 2500H, H gets data from 2501H

MOV A, H          : Move the first number (from H) into the Accumulator

SUB L                 : Subtract the second number (in L) from the Accumulator (A = A - L)

JNC STORE        : If Carry flag (Borrow) is NOT set (JNC), jump to STORE

INR C                  : If Carry is set, increment Register C to 01H (indicates a borrow)

CMA                    : Find 1's complement of the Accumulator to calculate 2's complement magnitude

ADI 01H              : Add 01H to get the 2's complement representation of the negative result

STORE:

STA 2502H         : Store the subtraction result from Accumulator into memory location 2502H

MOV A, C          : Move the borrow status from Register C into the Accumulator

STA 2503H        : Store the borrow status into memory location 2503H

HLT                    : Halt the program execution